2025 – 05 – 19

Updated Joinery, as per client.

PROJECT:	G&I – Office Block

- Kitchenette area behind the Boardroom TV wall:

    - 4630 (L) x 550 (D) x 900 (H) – Height includes base height

    - 100mm (H) x 20mm (T) Eezi Qaurtz splash back  – Magnolia

    - 20mm Thick Eezi Qaurtz countertop  – Magnolia

    - Black grab handle – Length 200mm

    - 16mm Melamine framed with impact edging

    - Melamine to be Storm Grey

    - 100mm (H) Base to be plywood and covered with Super Black.

    - 4x drawers

    - 1 x single center shelf in the middle of the cupboards. (Spaces: Top & Bottom)

- Server area in kitchenette area behind boardroom TV wall:

    - 2065 (L) x 350 (D) x 900 (H) – Height includes base height

    - 100mm (H) x 20mm (T) Eezi Qaurtz splash back  – Magnolia

    - 20mm Thick Eezi Qaurtz countertop  – Magnolia

    - Black grab handle – Length 200mm

    - 16mm Melamine framed with impact edging - Dunblane

    - Melamine doors - Dunblane

    - 100mm (H) Base to be plywood and covered with Iceberg White.

    - 1 x single center shelf in the middle of the cupboards. (Spaces: Top & Bottom)

    - Floating shelf:

      - 3 x black L-brackets fixed to drywall.

      - 32mm (T) melamine with impact edging.

      - Colour to be Brookhill Fusion

- Boardroom Table:

    - 4000 (L) x 1200 (D) x 770 (H) – Height includes base height

    - 50mm build up with 16mm Melamine and covered with Oak veneer to match wallpaper wall.

    - 20mm solid oak edging

    - 50mm x 70mm powder coated black steel base

    - Power docking allowance -  PC of R 4000.00

- 1 x Slatted  Bench:

- Solid oak slats as per design

- Mild steel sub frame

- Sanding down of wall panels in Boardroom + The frame and backing of the wallpaper:

    - Mural Frame: 5450 (L) x 1970 (H)

    - 16mm Backing for wallpaper

    - 32mm x 32mm MFD frame around backboard

    - 150mm (H) skimmed bulkhead to hide existing sprinkler system piping.

    - Painting and sanding of current wall panels (E.g.  panel size - 600x 2320mm)

(Needs to match boardroom table, else we need to make the wall panels oak veneer. As per our discussion.)

- Arch wall – Display area:

- 200mm high skimmed bulkhead

- Arch cover panel 18mm shutter-ply (2630mm (H) x 4151 mm (arched length))

- 150mm (H) shutter-ply base + Painted/Sprayed (Either or)

- Exhibition space (Display) – VOIDED

PC = R 8200

- Kitchenette on First Floor:

- Replace doors, draw fronts (Re-using existing carcasses only)

- Install new hinges and runners

- Install new base cladding

- Black grab handle – Length 200mm

- Remedial repair work to existing cabinets

- Install Eezi quarts 20mm top -  Magnolia

- 4 x floating shelves:

      - 1380mm (L) x 300mm (D) x 32mm (H)

      - 32mm thick (16x2mm) melamine with matching impact edging.

      - Colour to be Brookhill Fusion

2025 – 05 -06

As per meeting on site with client. It was asked to use the existing carcass as a template to install a complete new kitchenette.

Please include:  New framed area for wallpaper at bench

2140mm (w) x 1580mm (h) - To match boardroom frame style and colour.