BLACKHOUND ENTERPRISES PTY LTD

T/A BLACKHOUND ARCHITECTURE & RENDERING

Dewald Potgieter

dewald@blackhoundenterprises.com

072 418 6867

Unit 10 – The Network

19 Cactus Road

Fairview

Port Elizabeth (Gqeberha)

Eastern Cape

2075

Unit 10 – The Network

19 Cactus Road

Fairview

Port Elizabeth (Gqeberha)

Eastern Cape

2075

Unit 10 – The Network

19 Cactus Road

Fairview

Port Elizabeth (Gqeberha)

Eastern Cape

2075

HINTERVELD – REV00					                 	       2025-08-05

SNAGLIST (Prelim – Not Final Completion list):

General:

Lot of cornice junctions not looking good. Some needs sanding. The recess between the cornices needs to be either brilliant white or kitten white. I see some painted both colours.

I noticed a lot of patches on the walls. We need to repaint these walls. There are also instances where we can see some of the undercoat at certain areas.

All cupboards too be checked. I noticed a lot of door handles catching.

Check all cupboards as there are some missing shelves, edge trims and cover strips.

All steel frames too be investigated, as I have noticed at least two steel frames where rust is coming through the paint.

Existing roller-catch to be adjusted on all the existing doors.

Cleaning. A lot of surfaces are not cleaned, which we cannot check.

Internal:

- Patio

- They are still busy painting the ceiling. Decking cannot be checked.

- Foyer/Entrance

- Front door floor pin not working.

- Access control is not working and has not been coded.

- Pop up plug not coming out.

- Wall papers not installed.

- Pop up plug to be moved as you can still see the cutout on the timber floor.

- Showroom

- Some cupboard handles are catching.

- Check cornices as per general comment.

- Door and hatch not installed.

- Wall papers not installed.

- Plug protruding too far out the wall.

- Marks on ceiling and cornice. Looks like ding marks.

- Light switch edges not neatly finished off.

- Passage 1

- Skirtings to be sealed at the bottom in the passage.

- Grouting missing, please check the area.

- Check cornices as per general comment.

- Timber floorboard bulge at tile/timber transition.

- Canteen

- Cleaning. I cannot see the countertop so lekker. Also, a lot of marks on the cupboard doors, might just need a clean before we can comment.

- Water was turned off. Still needs to be tested.

- Female restroom

- Opaque vinyl to be added to windows.

- Toilet paper holder not installed.

- Mirror not installed.

- Steel frames to be filled and leveled out. I still see marks on the existing frame.

- New chipped marks on the painted frame

- Sanitaryware - Water was turned off. Still needs to be tested

- Paper hand towel dispenser has not been reinstated yet.

- Male restroom

- Opaque vinyl to be added to windows.

- Toilet paper holder not installed.

- Mirror not installed.

- Steel frames to be filled and leveled out. I still see marks on the existing frame.

- New chipped marks on the painted frame

- Sanitaryware - Water was turned off. Still needs to be tested.

- Inspect door handle as it gets stuck.

- Paper hand towel dispenser has not been reinstated yet.

- Coffee Nook

- Wall not finished painted at plug below counter.

- Grouting missing on tiles.

- Marks on wall at plug.

- Chipped marks underneath shelf.

- Sanitaryware - Water was turned off. Still needs to be tested.

- Require an end cap on hot water pipe. (hot water boiler provision)

- Visitors Restroom

- Opaque vinyl to be added to windows.

- Toilet paper holder not installed.

- Mirror not installed.

- Steel frames to be filled and leveled out. I still see marks on the existing frame.

- New chipped marks on the painted frame

- Install door spacer. Too much movement on the closed toilet door.

- Paper hand towel dispenser has not been reinstated yet.

- Reception

- Missing plug on counter. Willie will get it installed as per our discussion on site.

- Missing shelf

- Back wall to be cleaned from silicone.

- Wall unfinished and still need to be painted.

- Passage 2

- Plug protruding too far out the wall.

- Server/Storage

- Door stops to be added.

- Motion detection to be installed.

- Office 2

- Thumb-turn to be flipped to Daniel’s side.

- Chip marks on one of the tops of desks

- Black cam-locks to be installed on vertical cupboards.

- Handles catching on vertical cupboards.

- Daniels Office

- Scratch marks on the countertop of the wall unit next to Daniels desk.

- Vent cover plate not installed at fridge position.

- Check the top drawer at Daniel’s desk.

- Office 1

- Ac unit to be reinstated and condenser to be positioned around the corner and not in the front façade.

- Check the top drawer at Sharon’s desk.

- Check bottom drawer on the wall cupboards, as some of them are catching.